Perfect. I've read enough of the paper to provide a detailed research analysis. Going forward, I'll keep the same format for every paper you send.

Paper 1

Paper Name

DeepCAD: A Deep Generative Network for Computer-Aided Design Models

Authors: Rundi Wu, Chang Xiao, Changxi Zheng

Conference: ICCV 2021

Institution: Columbia University

Problem Statement

Most existing deep learning models generate 3D objects as point clouds, meshes, or voxels. These representations are suitable for visualization but cannot be directly edited in CAD software, making them impractical for engineering and manufacturing applications.

The authors aim to generate editable CAD models instead of discrete 3D representations.

Core Idea

Instead of generating a mesh, the model generates the sequence of CAD operations that a designer would perform.

For example,

Sketch Circle

↓

Extrude

↓

Boolean Union

↓

Final CAD Model

The CAD model is treated similarly to a sentence in natural language.

Each CAD command is considered a "word."

Therefore, the authors employ a Transformer-based Autoencoder to learn CAD command sequences.

Logic Behind the Principle

Step 1 — CAD Representation

A CAD object is converted into commands.

Example:

Sketch

Line

Arc

Circle

Extrude

Boolean

Every command has parameters.

Example:

Line

(x,y)

Circle

(center,radius)

Extrude

(distance, direction)

Instead of learning geometry directly,

the model learns

CAD Commands

+

Command Parameters

This is easier because CAD software already represents objects this way.

Step 2 — Convert Commands into Neural Input

Neural networks require fixed-size vectors.

The authors convert every command into

Command Type

+

16-dimensional Parameter Vector

Continuous values are quantized into 256 discrete levels.

Why?

Regression introduces small numerical errors that can destroy CAD constraints (parallel edges, perpendicular faces, etc.).

Classification into discrete bins preserves geometric relationships better.

Step 3 — Transformer Encoder

Each command is embedded.

Embedding includes

- Command type

- Parameters

- Position in sequence

Transformer Encoder →

latent vector (256 dimensions)

This latent vector becomes the compressed representation of the CAD model.

Step 4 — Transformer Decoder

The decoder receives

Latent Vector

↓

Transformer

↓

CAD Commands

↓

CAD Model

Thus,

the network reconstructs the original CAD operations.

Step 5 — Random CAD Generation

After training,

they train a GAN on the latent vectors.

Random Noise

↓

GAN

↓

Latent Vector

↓

Transformer Decoder

↓

New CAD Model

Thus the system can generate completely new CAD models.

Dataset

Existing CAD datasets were too small.

Therefore the authors built a dataset containing

178,238 CAD models

collected from Onshape.

Each model contains the full CAD construction sequence.

This is one of the major contributions of the paper.

Experimental Results

The paper reports:

- 99.5% command accuracy

- 97.98% parameter accuracy

- Low invalid CAD generation rate (~2.7%)

- Generated CAD models are editable in commercial CAD software.

Strengths

1. First deep model to generate editable CAD programs

Unlike meshes,

output can directly open inside CAD software.

2. Transformer captures sequential CAD operations

CAD modeling is naturally sequential.

Transformer fits this representation very well.

3. Large Dataset

178k CAD models significantly outperform previous CAD datasets in scale.

4. High Reconstruction Accuracy

Very low reconstruction error.

5. Editable Output

Generated models remain editable.

This is extremely valuable in engineering.

Limitations

1. Limited CAD Operations

Supports only

- Line

- Arc

- Circle

- Extrude

- Boolean

No support for

- Fillet

- Chamfer

- Loft

- Sweep

- Shell

- Revolve

- Complex free-form surfaces

2. No Semantic Understanding

The model only understands geometry.

It cannot answer

"Convert this chair into a hospital chair."

There is no understanding of object function or context.

3. No Natural Language Interface

Input is always

CAD Commands

not

Text Prompt

4. No Feature Compression

Entire CAD command sequence must be stored.

There is no compact representation like QR.

5. Cannot Modify Existing Objects Based on Context

It only reconstructs

or randomly generates.

No intelligent redesign.

6. Topological Errors Still Exist

Some generated CAD sequences become invalid.

The paper acknowledges this limitation.

How Your Method is Better

Your Proposed Pipeline

CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

User Prompt

↓

LLM / Generative AI

↓

Synthetic CAD Model

| DeepCAD | Your Method |
| --- | --- |
| Generates CAD command sequences | Generates context-aware synthetic CAD models |
| Uses Transformer only | Uses LLM/Generative AI for semantic reasoning |
| Requires full CAD sequence | Stores compressed geometric features in a QR code |
| No semantic understanding | Understands user intent (e.g., "hospital chair") |
| No natural language support | Supports natural language queries |
| Only reconstructs or generates random CAD | Reconstructs and adapts CAD models based on context |
| Geometry only | Combines geometry with semantic information |
| No portable encoding | QR acts as a lightweight, shareable representation |

Research Gap

This paper focuses on how to generate CAD programs, but it does not address:

- efficient storage or transfer of CAD knowledge,

- semantic feature encoding,

- natural language–guided design modification,

- or context-aware CAD synthesis.

Your MTP proposes to bridge these gaps by introducing a QR-encoded feature representation and using generative AI to reconstruct and adapt CAD models according to user requirements.

Overall Summary

DeepCAD is a foundational paper because it demonstrates that CAD models can be represented as sequences of operations and generated using Transformer architectures. Its main contribution is moving from mesh-based generation to editable CAD generation. However, it is limited to geometric reconstruction and lacks semantic reasoning, natural language interaction, and compact feature encoding. Your proposed MTP extends this direction by combining CAD slicing, QR-based feature encoding, and generative AI to enable context-aware, natural language–driven synthetic CAD reconstruction.

Paper 2

Paper Name

ORTHO2CAD: 3D CAD Generation from Orthographic Drawings using Vision Language Models

Authors: Aditya Joglekar, Amit Regmi, Kenji Shimada, Levent Burak Kara

Institution: Carnegie Mellon University (2026)

Problem Statement

In industry, engineers usually exchange 2D orthographic drawings rather than editable CAD models because drawings are easier to share and protect intellectual property.

However, manufacturing, simulation, and design modification require editable 3D CAD models.

Currently, engineers manually recreate the CAD model from these drawings, which is time-consuming.

The paper aims to automatically reconstruct an editable CAD model from rasterized orthographic drawings.

Core Idea

The paper converts

Orthographic Drawing

↓

Vision Language Model (Qwen3-VL)

↓

CadQuery Code

↓

Python Interpreter

↓

Editable CAD Model

Instead of predicting meshes or point clouds, the model predicts CadQuery code, which can be executed to generate a STEP/CAD model. It combines:

- Vision-Language Models (VLMs)

- Supervised Fine-Tuning (SFT)

- Reinforcement Learning (RL)

to improve CAD reconstruction.

Logic Behind the Principle

Step 1 — Generate Orthographic Drawings

Since no large dataset exists, the authors generate synthetic engineering drawings from STEP files.

Each drawing contains:

- Front View

- Top View

- Side View

- Hidden lines

- Basic dimensions

using pythonOCC.

Step 2 — Vision Language Model

The input is

Orthographic Drawing

+

Prompt

↓

Qwen3-VL

Prompt example:

Generate the CadQuery code needed to create the CAD.

The VLM learns the relationship between drawing features and CAD commands.

Step 3 — Supervised Fine-Tuning (SFT)

For datasets that already contain CadQuery code (DeepCAD),

they train using

Drawing

↓

Ground Truth CadQuery Code

↓

Cross Entropy Loss

The objective is to predict the exact CAD program.

Step 4 — Reinforcement Learning

Most CAD datasets don't contain CadQuery code.

Instead of comparing code,

they compare geometry.

Pipeline:

Drawing

↓

Generated CadQuery

↓

Execute Code

↓

3D Model

↓

IoU with Ground Truth

↓

Reward

Higher IoU → higher reward.

Lower IoU → lower reward.

This allows learning without ground-truth code.

Step 5 — Final CAD Reconstruction

The generated CadQuery program is executed to produce

Editable STEP Model

which can be modified in CAD software.

Dataset

The authors generate

150,000+ orthographic drawings

using:

- DeepCAD

- Fusion360

Each drawing contains:

- Three orthographic views

- Hidden lines

- Bounding-box dimensions

This is one of the major contributions of the paper.

Experimental Results

DeepCAD Dataset

- 100% valid CadQuery code

- IoU = 0.7922

- 7.6% improvement over CAD-Coder

Fusion360 Dataset

After RL:

- 100% executable code

- IoU = 0.5601

- Better than GPT-5.2 and CAD-Coder baselines.

Strengths

1. Uses Real Engineering Drawings

Unlike previous works using perspective images,

this paper uses

- Front view

- Side view

- Top view

which aligns with industrial workflows.

2. Produces Editable CAD

Output is CadQuery code,

not meshes.

3. Reinforcement Learning

Can learn even without ground-truth CAD code.

Very practical.

4. Large Dataset

150k+ orthographic drawings.

5. High CAD Validity

100% executable code.

Limitations

1. Requires Complete Orthographic Drawings

The system depends on

- front view

- side view

- top view

If any view is missing,

performance drops.

2. No Semantic Understanding

The model only reconstructs geometry.

It cannot understand

"Convert this chair into a hospital chair."

3. No Design Adaptation

Input

↓

Output

is almost one-to-one.

It reconstructs.

It does not redesign.

4. No Compact Representation

The whole drawing must be supplied every time.

No compressed representation such as QR.

5. Limited CAD Operations

Dataset mainly contains

- Sketch

- Extrude

Complex operations like

- Fillet

- Loft

- Shell

- Sweep

are mostly absent.

6. Still Struggles with Thin Structures

The paper itself shows

- thin plates

- thin rods

- hollow structures

often have low IoU.

How Your Method is Better

Your Proposed Pipeline

CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

User Prompt

("Hospital Chair")

↓

LLM

↓

Synthetic CAD Model

| Ortho2CAD | Your Method |
| --- | --- |
| Requires three orthographic drawings | Requires only a QR code containing extracted features |
| Reconstructs existing CAD | Reconstructs and redesigns CAD |
| No semantic understanding | Uses natural language prompts to understand design intent |
| Geometry only | Combines geometry + semantics |
| Needs complete drawings every time | Uses a compact QR representation for storage and transfer |
| Produces one reconstruction | Generates multiple context-aware synthetic CAD variants |
| No object adaptation | Can modify the object for new domains (hospital, office, classroom, etc.) |
| Dependent on orthographic drawings | Works from QR-encoded features plus user intent |

Research Gap

Although Ortho2CAD successfully reconstructs editable CAD models from orthographic drawings, it still treats reconstruction as a geometry translation task. It does not encode object semantics, compress design knowledge into a portable representation, or support context-aware redesign through natural language. Your MTP addresses these gaps by introducing QR-based feature encoding and combining it with LLMs to generate synthetic, context-aware CAD models (e.g., adapting a generic chair into a hospital chair while preserving essential structural characteristics).

Overall Summary

Ortho2CAD is one of the first works to reconstruct editable CAD models directly from engineering orthographic drawings using Vision-Language Models, supervised fine-tuning, and reinforcement learning. Its major contribution is bridging 2D engineering drawings and executable CadQuery programs. However, it remains focused on reconstructing existing geometry and does not perform semantic design transformation or compact feature encoding. Your proposed MTP extends this direction by representing CAD features in a QR code and leveraging generative AI to create new, context-aware CAD designs from user intent while preserving the core geometry.

Paper 3

Paper Name

GenCAD-3D: CAD Program Generation using Multimodal Latent Space Alignment and Synthetic Dataset Balancing

Authors: Nomi Yu, Md Ferdous Alam, A. John Hart, Faez Ahmed

Institution: Massachusetts Institute of Technology (MIT)

Journal: Journal of Mechanical Design (2025 Preprint)

Problem Statement

Modern AI models can generate meshes, point clouds, and 3D shapes, but engineers require editable CAD programs rather than static geometry.

Reverse engineering currently involves:

Physical Object

↓

3D Scan

↓

Manual CAD Reconstruction

↓

Editable CAD

This process is:

- Time consuming

- Requires CAD experts

- Difficult for complex geometries

Another challenge is that existing datasets (DeepCAD) contain mostly simple CAD models, causing AI models to perform poorly on complex parts.

Core Idea

Instead of directly generating CAD from geometry,

GenCAD-3D first learns a shared latent space between:

- CAD Programs

- Point Clouds

- Meshes

using Contrastive Learning.

Then it uses a Latent Diffusion Model to convert geometric representations into CAD programs.

Finally,

they introduce SynthBal, a synthetic dataset balancing strategy that improves learning on complex CAD programs.

Logic Behind the Principle

The complete pipeline consists of three stages.

Stage 1 — Learn CAD Representation

Every CAD model is represented as

Sketch

↓

Extrude

↓

Sketch

↓

Extrude

Every operation contains

- Command

- Parameters

Example

Circle

Radius

Center

↓

Extrude

Distance

These commands are converted into a

60 × 17 matrix

where

- first column = command

- remaining 16 = parameters.

A Transformer Autoencoder compresses this into a latent vector representing the CAD model.

Stage 2 — Multimodal Latent Space Alignment

This is the novelty of the paper.

Instead of only learning CAD,

the model learns a common embedding space for

Mesh

Point Cloud

CAD Program

using Contrastive Learning.

The idea:

If

Mesh A

↓

Chair

and

CAD A

↓

Same Chair

their latent vectors should be close.

If

Mesh A

↓

Chair

and

CAD B

↓

Table

their latent vectors should be far apart.

Positive pairs are pulled together,

negative pairs pushed apart using cosine similarity loss.

Stage 3 — Conditional Latent Diffusion

After learning the shared latent space,

the model trains a diffusion model.

Pipeline:

Mesh

↓

Encoder

↓

Mesh Latent

↓

Diffusion

↓

CAD Latent

↓

Transformer Decoder

↓

CAD Program

Instead of predicting CAD directly,

it predicts the CAD latent vector,

which is easier to learn.

Finally,

Transformer Decoder reconstructs the CAD commands.

Stage 4 — SynthBal Dataset

Existing DeepCAD dataset

178k CAD models

contains mostly

Simple CAD.

Complex CAD models are rare.

Therefore,

the authors generate synthetic CAD programs.

Two augmentation methods are used:

Noise

Small perturbations are added to CAD parameters.

Example

Radius

20

↓

21

Replace Sketch

Randomly replace sketch operations with sketches from another CAD model.

Example

Chair Seat

↓

Different Chair Seat

Only valid CAD models are kept.

The final balanced dataset contains

- 170k samples

- 1 Million samples

depending on training configuration.

Architecture

Mesh

│

▼

Mesh Encoder

│

▼

Mesh Latent Vector

│

▼

Contrastive Alignment

│

▼

Shared Latent Space

│

▼

Conditional Diffusion

│

▼

CAD Latent Vector

│

▼

Transformer Decoder

│

▼

CAD Program

│

▼

Editable CAD Model

Dataset

The paper extends DeepCAD into multiple modalities.

Training data includes:

- 168,656 valid CAD programs

- Mesh dataset

- Point cloud dataset

- Point cloud + normals dataset

Additionally,

they generate

- SynthBal (170k)

- SynthBal-1M (1 million)

synthetic datasets for balancing complex CAD sequences.

Experimental Results

Autoencoder Performance

The SynthBal-1M FT model achieved:

- Command Accuracy: 99.66%

- Parameter Accuracy: 98.61%

- IoU: 96.5%

- Invalid CAD Ratio: 0.807%

Retrieval Performance

Using mesh embeddings,

Top-1 retrieval accuracy:

- 99.7% (library size = 10)

- 97.2% (128)

- 87.9% (1024)

- 81.6% (2048)

Conditional CAD Generation

Best model:

- Command Accuracy: 60.48%

- Parameter Accuracy: 52.64%

- Better reconstruction using mesh than point clouds.

Strengths

1. First multimodal latent alignment

Aligns

- Mesh

- Point cloud

- CAD

inside one embedding space.

2. Better Reverse Engineering

Supports

3D Scan

↓

Editable CAD

instead of manual reconstruction.

3. SynthBal

Introduces a novel synthetic dataset balancing strategy.

This significantly improves learning on complex CAD programs.

4. Diffusion-Based CAD Generation

Uses latent diffusion instead of directly predicting CAD commands.

This provides more robust conditional generation.

5. Supports Multiple Input Modalities

Unlike previous work,

supports

- Mesh

- Point Cloud

- Point Cloud + Normals

6. Large-scale Dataset

Uses

- 168k CAD models

- 1 million synthetic CAD samples

Limitations

1. No Natural Language Understanding

Input must be

- Mesh

- Point cloud

It cannot understand

"Convert this chair into a hospital chair."

2. No Semantic Information

The model only learns geometry.

It has no knowledge about

- hospital

- office

- classroom

- ergonomic design

3. QR Encoding Not Used

Entire latent representation must be stored.

No lightweight transferable representation.

4. Limited CAD Commands

Although the architecture can be extended, experiments mainly use:

- Line

- Arc

- Circle

- Extrude

Complex industrial operations such as fillet, sweep, loft, revolve, and shell are not included in the experiments.

5. Moderate Reconstruction Accuracy

Even the best conditional model achieves around

- 60% command accuracy

- 53% parameter accuracy

showing that geometry-to-CAD reconstruction remains challenging.

6. High Computational Cost

The framework trains:

- Transformer Autoencoder

- Contrastive Encoder

- Diffusion Model

and uses H100 GPUs with large synthetic datasets, making training expensive.

How Your Method is Better

Your Proposed Pipeline

CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM + Generative AI

↓

Synthetic CAD Model

| GenCAD-3D | Your MTP |
| --- | --- |
| Requires mesh or point cloud | Uses QR-encoded features from a CAD model |
| Focuses on reverse engineering | Focuses on context-aware CAD redesign and generation |
| No natural language interface | Supports prompts such as "Design this chair for a hospital" |
| Learns only geometric embeddings | Combines geometric features with semantic understanding |
| Stores high-dimensional latent vectors | Uses a compact QR code for storage and sharing |
| Generates CAD similar to the input | Generates modified, application-specific CAD models |
| Limited to reconstruction | Supports reconstruction and intelligent redesign |

Research Gap

GenCAD-3D demonstrates that multimodal latent alignment can bridge meshes, point clouds, and CAD programs, but it remains a geometry-centric approach. It does not encode design semantics, provide a compact transferable representation, or enable user-driven design adaptation through natural language. Your MTP addresses these gaps by introducing QR-based feature encoding and using LLMs to generate context-aware CAD models that preserve core geometry while adapting to new application domains.

Overall Summary

GenCAD-3D is a significant advance in automated CAD generation by combining multimodal latent space alignment, contrastive learning, latent diffusion, and synthetic dataset balancing. It improves reverse engineering from meshes and point clouds into editable CAD programs and achieves strong reconstruction and retrieval performance. However, its focus remains on reconstructing geometry rather than understanding design intent. Your proposed MTP extends this direction by integrating QR-encoded geometric features with natural language-driven generative AI, enabling compact representation, semantic reasoning, and application-specific synthetic CAD generation.

Paper 4

Paper Name

Text-to-CadQuery: A New Paradigm for CAD Generation with Scalable Large Model Capabilities

Authors: Haoyang Xie, Feng Ju

Institution: Arizona State University

Year: 2025 (Preprint)

Problem Statement

Current Text-to-CAD methods generate intermediate CAD command sequences instead of directly generating editable CAD models. These command sequences:

- Require custom neural architectures.

- Need conversion into CAD vectors or JSON before creating a CAD model.

- Require training models from scratch.

- Increase computational complexity.

The authors ask:

"Why generate intermediate command sequences at all?"

Instead, they propose generating CadQuery code directly from natural language.

Core Idea

The paper replaces the traditional CAD generation pipeline with a direct Text → CadQuery approach.

Instead of

Text

↓

CAD Commands

↓

JSON

↓

CAD Model

they propose

Text

↓

Large Language Model

↓

CadQuery Python Code

↓

Execute

↓

Editable CAD Model

Since CadQuery is a Python-based CAD scripting language, pretrained LLMs already possess Python coding knowledge, making it easier to fine-tune them for CAD generation.

Logic Behind the Principle

The paper consists of four major stages.

Stage 1 — Build a Large Dataset

The authors extend the Text2CAD dataset.

Original dataset:

- Natural language descriptions

- CAD command sequences

They add:

- CadQuery code for every CAD model.

Result:

Text Prompt

↓

CadQuery Code

Approximately 170,000 text–CadQuery pairs are created.

Stage 2 — Automatic CadQuery Annotation

Instead of manually writing CadQuery code,

they use Gemini 2.0 Flash.

Pipeline:

Minimal JSON

↓

Gemini

↓

CadQuery Code

↓

Execute

↓

STL Model

↓

Render Image

↓

Gemini Validation

↓

Human Verification

If execution fails,

Gemini receives the error message and corrects the code.

This iterative refinement improves execution success from 53% to 85% during annotation.

Stage 3 — Fine-Tune Large Language Models

Six pretrained LLMs are fine-tuned:

- CodeGPT-small (124M)

- GPT-2 Medium

- GPT-2 Large

- Gemma-3-1B

- Qwen2.5-3B

- Mistral-7B (LoRA)

Instead of training from scratch,

they only fine-tune existing models.

Stage 4 — Generate CAD

Input:

"Generate a cylindrical container."

↓

LLM

↓

import cadquery as cq

result = cq.Workplane("XY").circle(...).extrude(...)

↓

Execute Python

↓

Editable CAD Model

The generated Python code directly creates the CAD model without any intermediate CAD command representation.

Architecture

Natural Language

↓

Tokenizer

↓

Pretrained LLM

↓

CadQuery Python Code

↓

Python Execution

↓

Editable CAD Model

This architecture is much simpler than previous Text2CAD pipelines because it removes the command-sequence decoding stage.

Dataset

The dataset is built from:

- DeepCAD

- Text2CAD

and extended with approximately:

- 170,000 text–CadQuery pairs

- 600,000+ natural language prompts

This is currently one of the largest Text-to-CAD datasets using executable CadQuery scripts.

Experimental Results

The best-performing model (Qwen2.5-3B) achieved:

- 69.3% Top-1 Exact Match

- 48.6% reduction in Chamfer Distance

- 10.5% improvement over the Text2CAD baseline

The Mistral-7B model produced the lowest invalid code rate (1.32%), showing excellent code executability, while Qwen2.5-3B achieved the best overall reconstruction accuracy.

Strengths

1. Direct Text-to-CAD Generation

No intermediate CAD command representation.

Simpler pipeline.

2. Uses Existing LLMs

No need to design custom Transformer architectures.

Leverages pretrained Python knowledge.

3. Large Dataset

170k paired samples.

4. Faster Training

Smallest model trains in

58 minutes

compared to

2 days

for Text2CAD.

5. Better CAD Accuracy

Produces better CAD reconstruction than Text2CAD across Chamfer Distance, IoU, and F1 metrics.

6. Scalable

Performance improves consistently as model size increases (up to Qwen2.5-3B), demonstrating the benefits of scaling pretrained LLMs for CAD generation.

Limitations

1. Only Generates from Text

Input must be a natural language description.

It cannot use:

- Existing CAD models

- QR codes

- Point clouds

- Feature descriptors

2. No Feature Preservation

If a chair already exists,

the model does not preserve its geometric identity.

Every design is generated solely from text.

3. No Context-Based Redesign

The system cannot perform:

Existing Chair

↓

Hospital Chair

It only creates CAD from text.

4. No Compact Representation

Entire CAD program must be regenerated every time.

No QR encoding.

5. Dependent on Training Data

Performance drops for larger models because

170k samples are still relatively small for 7B models.

The authors explicitly mention that larger datasets (13B, 32B models) would likely improve results.

6. No Semantic Memory

Generated CAD does not preserve:

- Functional purpose

- Material

- Usage domain

It only follows the text prompt.

How Your Method is Better

Your Proposed Pipeline

Existing CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic CAD Model

| Text-to-CadQuery | Your MTP |
| --- | --- |
| Input is only natural language | Uses an existing CAD model plus user intent |
| Generates CAD from scratch | Reconstructs and modifies existing designs |
| No geometric feature preservation | Preserves extracted geometric and structural features |
| No compact representation | Encodes features into a lightweight QR code |
| Text-only generation | Combines QR features with natural language |
| No context-aware adaptation | Can redesign objects for new environments (e.g., hospital chair) |
| Produces one CAD model | Can generate multiple synthetic design variants while retaining key characteristics |

Research Gap

The paper successfully demonstrates that CadQuery is a superior output representation for text-to-CAD generation, replacing task-specific command sequences with executable Python code. However, it assumes that natural language alone is sufficient to describe a design. It does not support reconstructing or modifying an existing CAD model, preserving geometric features, or storing design information in a transferable format. Your MTP addresses these limitations by introducing a QR-based feature representation that captures geometric information from an existing CAD model and combines it with LLM-driven semantic reasoning to generate context-aware synthetic CAD models.

Overall Summary

Text-to-CadQuery presents a new paradigm for CAD generation by directly translating natural language into executable CadQuery scripts using pretrained LLMs. By eliminating intermediate CAD command sequences, the method simplifies the generation pipeline, reduces training time, and improves reconstruction quality. However, it remains a text-only generative system and lacks mechanisms for preserving or adapting existing designs. Your proposed MTP extends this concept by integrating QR-encoded geometric features with natural language prompts, enabling not only CAD generation but also feature-preserving, context-aware redesign of existing CAD models.

Great. Since these are research papers, I'll analyze them one by one in the same detailed format as before.

Paper 5

Paper Name

GaussianCAD: Robust Self-Supervised CAD Reconstruction from Three Orthographic Views Using 3D Gaussian Splatting

Authors: Zheng Zhou et al.

Journal: Elsevier (Preprint 2025)

Institution: Beijing University of Posts and Telecommunications, China

Problem Statement

Most CAD reconstruction methods require

- Vector CAD sketches

- Ground-truth 3D CAD models

Both are difficult to obtain in real industrial applications.

Additionally,

traditional reconstruction methods fail when

- sketches contain noise

- endpoints shift

- drawings are scanned

because they depend on exact geometric coordinates.

The authors aim to reconstruct 3D CAD models directly from raster orthographic drawings without requiring vector sketches or ground-truth 3D models.

Core Idea

The paper introduces an entirely different viewpoint.

Instead of treating CAD reconstruction as

Sketch

↓

CAD Generation

they formulate it as

Orthographic Views

↓

Sparse-view 3D Reconstruction

↓

3D CAD

They adapt 3D Gaussian Splatting (3DGS), originally developed for natural-image 3D reconstruction, to work with engineering drawings.

The key innovation is treating CAD reconstruction as a special case of sparse-view reconstruction.

Logic Behind the Principle

The proposed framework consists of four major stages.

Stage 1 — Sketch Filtering

Input:

Front View

Left View

Bottom View

These drawings contain

- visible lines

- hidden (dashed) lines

- noise

The first module

- removes dashed lines

- detects visible contours

- removes unwanted edges

Result

Clean Sketch

This makes the reconstruction process more robust.

Stage 2 — Sketch Colorization

Unlike photographs,

CAD sketches only contain lines.

3D Gaussian Splatting expects

- textures

- colors

Therefore,

the authors convert the sketches into pseudo-images.

Pipeline

Detected Boundary

↓

Foreground Mask

↓

Fill Interior

↓

Natural-image-like Representation

Now the input resembles an image that can be processed by Gaussian Splatting.

Stage 3 — Camera Pose Estimation

3DGS requires accurate camera parameters.

Natural images estimate them automatically.

CAD sketches cannot.

Therefore,

the paper manually computes

- intrinsic matrix

- world-to-camera (w2c) matrix

- rotation matrix

for

- Front View

- Left View

- Bottom View

These matrices correctly position each orthographic projection in 3D space.

Stage 4 — Gaussian Splatting Reconstruction

After preprocessing,

the three processed images are fed into

GaussianObject

(a modified 3D Gaussian Splatting framework).

Pipeline

Three Views

↓

Gaussian Initialization

↓

Optimization

↓

3D Gaussian Model

↓

CAD Reconstruction

Unlike NeRF,

3DGS represents the object as

thousands of

3D Gaussian Points

Each point stores

- position

- color

- opacity

- covariance

These Gaussian blobs reconstruct the entire object efficiently.

Architecture

Orthographic Drawings

↓

Sketch Filtering

↓

Mask Generation

↓

Colorization

↓

Camera Pose Calculation

↓

3D Gaussian Splatting

↓

3D CAD Model

Dataset

Experiments were conducted on

Sub-Fusion360

derived from

Fusion360.

Dataset contains

- Thousands of CAD sketches

- Corresponding CAD models

Only

three orthographic views

are used.

Experimental Results

Compared with

- GaussianObject

- Photo2CAD

GaussianCAD achieved

- Better reconstruction quality

- Higher robustness to noisy sketches

- State-of-the-art reconstruction accuracy

The qualitative comparisons in Figure 1 show noticeably cleaner and more accurate reconstructions across different CAD objects.

Strengths

1. No Ground Truth Needed

Completely

Self-supervised.

No 3D labels.

2. Uses Raster Sketches

No vector drawings.

Practical for industry.

3. Robust to Noise

Traditional methods fail.

GaussianCAD still reconstructs.

4. Uses Only Three Views

Unlike NeRF,

requires only

Front

Left

Bottom

5. Uses Existing 3DGS

No need to build an entirely new reconstruction network.

6. High-quality Visualization

3DGS provides realistic visualization and efficient rendering of reconstructed models.

Limitations

1. Requires Three Orthographic Views

Cannot reconstruct from

- one image

- QR

- point cloud

2. No Semantic Understanding

It reconstructs

geometry only.

Cannot answer

Make this chair suitable for hospital use.

3. No CAD Editing

Produces

3D reconstruction

not

editable CAD operations.

4. No Natural Language Interface

Input

↓

Sketch

only.

No LLM.

5. No Compact Representation

Entire sketches must always be transmitted.

No QR.

6. Manual Camera Calibration

Camera poses are manually computed.

Large-scale deployment becomes difficult.

How Your Method is Better

Your Proposed Pipeline

CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic CAD Model

| GaussianCAD | Your MTP |
| --- | --- |
| Uses three orthographic drawings | Uses QR-encoded CAD features |
| Requires sketch preprocessing | Uses directly extracted geometric features |
| Manual camera pose estimation | No camera calibration required |
| Reconstructs geometry | Reconstructs and redesigns geometry |
| No semantic understanding | Understands prompts like "hospital chair" |
| No natural language support | Uses LLM-guided generation |
| Large raster inputs | Compact QR representation |
| Reconstruction only | Context-aware synthetic CAD generation |

Research Gap

GaussianCAD effectively reconstructs 3D geometry from orthographic sketches without requiring vector data or ground-truth supervision. However, it remains focused on geometric reconstruction. It does not encode object semantics, support natural-language-driven modifications, or provide a lightweight mechanism for storing and sharing design information. Your MTP addresses these gaps by extracting CAD features into a QR code and combining them with LLM-based reasoning to generate application-specific synthetic CAD models while preserving the original design intent.

Overall Summary

GaussianCAD introduces a novel self-supervised approach that adapts 3D Gaussian Splatting for CAD reconstruction from raster orthographic sketches. By reformulating CAD reconstruction as a sparse-view reconstruction problem, it removes the dependency on vector sketches and 3D ground truth while achieving strong robustness to noisy inputs. Nevertheless, it is limited to geometry reconstruction and lacks semantic reasoning, compact feature encoding, and natural-language interaction. Your proposed MTP extends this direction by introducing QR-based feature encoding and LLM-guided context-aware CAD generation, enabling not only reconstruction but also intelligent redesign of CAD models.

Paper 6

Paper Name

Text2CSG: Generating CAD Models from Natural Language via Constructive Solid Geometry

Authors: Luo Zhang, Gaochao Song, Haocong Rao, Zhengyu Wen, Jiangbei Hu, Ying He

Journal: Computer-Aided Design (Elsevier), 2026

Problem Statement

Current Text-to-CAD methods have several limitations:

- They represent CAD as linear sketch-extrusion command sequences, making it difficult to model complex multi-part objects.

- They struggle to capture hierarchical construction logic and Boolean relationships.

- Existing methods often require fine-tuning large language models (LLMs), which is computationally expensive.

- Representations such as meshes, voxels, and point clouds do not explicitly preserve CAD construction semantics.

The authors aim to generate editable CAD models directly from natural language while preserving the construction hierarchy of CAD designs.

Core Idea

Instead of representing CAD models as a sequence of sketch and extrusion commands, the paper represents them using Constructive Solid Geometry (CSG).

A CAD model is expressed as a hierarchy of

- Primitive shapes

- Boolean operations

such as

Cube

Cylinder

Sphere

↓

Union

Difference

Intersection

↓

Final CAD Model

This hierarchical representation better reflects engineering design logic.

Logic Behind the Principle

The proposed framework consists of two stages.

Stage 1 — Structure-Aware CSG Program Generation

The CAD model is represented as a CSG Tree.

Example

Union

/     \

Cylinder   Difference

/        \

Cube      Cylinder

Instead of flattening the tree,

the authors preserve

- parent-child relationships

- sibling relationships

- Boolean hierarchy

using a structure-aware Transformer decoder.

Stage 2 — Natural Language Alignment

A frozen BERT language model extracts semantic information from the user's text.

Example

Prompt

↓

BERT

↓

Semantic Embedding

These embeddings are passed to the Transformer using cross-attention.

Unlike many approaches,

the LLM is not fine-tuned.

Only the CSG decoder is trained.

Stage 3 — Hierarchical Token Encoding

Each CSG tree is converted into tokens.

Every token contains

- Primitive type

- Boolean operator

- Parent index

- Depth information

Example

Cube

↓

Token

Cylinder

↓

Token

Union

↓

Token

This enables the Transformer to learn CAD hierarchy rather than a simple sequence.

Stage 4 — CSG Validity Loss

One major challenge is that generated CSG programs may become invalid.

To solve this,

the authors introduce a CSG Validity Loss.

The loss penalizes

- Invalid Boolean operators

- Incorrect hierarchy

- Broken CSG trees

Thus,

generated programs remain executable.

Stage 5 — Geometry Decoder

The generated CSG program is executed.

Pipeline

Natural Language

↓

CSG Program

↓

Geometry Decoder

↓

CAD Model

The final model is

- editable

- interpretable

- structurally meaningful.

Architecture

Natural Language

↓

Frozen BERT

↓

Semantic Embedding

↓

Cross Attention

↓

Structure-aware Transformer

↓

CSG Program

↓

Geometry Decoder

↓

CAD Model

Dataset

The authors construct a new

CSG Dataset

where every sample contains

- Natural language description

- CSG Tree

- CAD model

Unlike DeepCAD,

the dataset explicitly stores

hierarchical construction logic

instead of only sketch-extrusion sequences.

Experimental Results

Compared with

- DeepCAD

- Text2CAD

- CADmium

Text2CSG achieved

- Higher geometric accuracy

- Better structural consistency

- Lower invalid program generation

- Better reconstruction of complex multi-component CAD models

The paper attributes these gains to the explicit modeling of CSG hierarchy and the CSG Validity Loss.

Strengths

1. Hierarchical CAD Representation

Unlike sequential CAD,

uses

CSG Tree

which naturally represents engineering designs.

2. Better Complex Object Generation

Can model

- holes

- unions

- intersections

- multi-part objects

more naturally.

3. Frozen LLM

No expensive LLM fine-tuning.

Computationally efficient.

4. Structure-aware Transformer

Learns parent-child relationships.

Not just sequential information.

5. Low Invalid CAD Rate

The CSG Validity Loss ensures generated programs remain executable.

6. Editable CAD

Produces interpretable CSG programs,

not meshes.

Limitations

1. Only Generates from Text

Input

↓

Natural Language

It cannot use

- Existing CAD

- QR codes

- Point clouds

- Orthographic drawings

2. Primitive-Based Representation

The method primarily relies on analytic primitives and Boolean operations. Complex industrial free-form surfaces and advanced CAD features are not the focus of the representation.

3. No Existing CAD Preservation

Every CAD model is generated

from scratch.

Existing geometry cannot be reused.

4. No Feature Encoding

No lightweight storage mechanism

such as QR.

5. No Context-Aware Modification

Cannot answer

Convert this office chair into a hospital chair.

The model creates a CAD object from the prompt rather than adapting an existing one.

6. Limited to CSG Representation

CSG is excellent for Boolean-based designs but may not capture all modeling workflows used in modern commercial CAD systems, such as feature histories involving lofts, sweeps, or fillets.

How Your Method is Better

Your Proposed Pipeline

Existing CAD

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic CAD Model

| Text2CSG | Your MTP |
| --- | --- |
| Generates CAD from text | Modifies existing CAD intelligently |
| Uses CSG trees | Uses extracted geometric features from sliced CAD |
| No feature preservation | Preserves original geometry through QR encoding |
| No QR representation | Lightweight QR-based design transfer |
| No redesign capability | Supports context-aware redesign (e.g., hospital chair) |
| Generates one CAD model | Generates multiple synthetic design variants |
| Relies only on text | Combines geometry + semantic prompts |

Research Gap

Text2CSG demonstrates that hierarchical CSG programs are a more suitable representation than linear sketch-extrusion sequences for text-driven CAD generation. However, it assumes that the design starts entirely from a text description. It does not preserve information from an existing CAD model, encode geometric features into a compact transferable representation, or perform semantic adaptation of an existing object. Your MTP addresses these limitations by introducing QR-based feature encoding from an existing CAD model and combining it with LLM-based semantic reasoning to generate context-aware synthetic CAD models while maintaining key geometric characteristics.

Overall Summary

Text2CSG presents a novel framework for text-driven CAD generation by representing CAD models as Constructive Solid Geometry (CSG) trees instead of linear command sequences. Through a structure-aware Transformer, hierarchical token encoding, cross-attention with a frozen BERT model, and a CSG Validity Loss, it produces more accurate and structurally consistent CAD models than previous text-to-CAD approaches. However, it remains a text-only generation framework and does not support existing CAD reconstruction, feature preservation, or context-aware redesign. Your proposed MTP extends this direction by integrating QR-encoded CAD features with natural language prompts, enabling both preservation of existing geometry and intelligent, application-specific CAD generation.

Paper 8

Paper Name

Image2Gcode: Image-to-G-code Generation for Additive Manufacturing using Diffusion-Transformer Model

Authors: Ziyue Wang, Yayati Jadhav, Peter Pak, Amir Barati Farimani

Journal: Additive Manufacturing Letters (Elsevier), 2026

Problem Statement

The conventional 3D printing workflow consists of multiple stages:

Concept

↓

CAD Model

↓

STL File

↓

Slicing

↓

G-code

↓

3D Printing

This workflow has several drawbacks:

- Requires CAD expertise.

- Every design modification requires updating the CAD model.

- Multiple conversion steps increase complexity and processing time.

- Rapid prototyping becomes slow and designer-dependent.

The authors aim to eliminate the CAD modeling stage and generate printer-ready G-code directly from images or sketches.

Core Idea

Instead of generating a CAD model first, the paper directly predicts G-code from an image.

The proposed workflow is:

Image / Sketch

↓

Feature Extraction

↓

Diffusion Model

↓

G-code

↓

3D Printer

Thus,

the CAD and slicing stages are completely bypassed.

Logic Behind the Principle

The framework consists of three stages.

Stage 1 — Dataset Preparation

The model is trained using the Slice-100K dataset, containing over 100,000 aligned STL–G-code pairs.

Each sample contains:

- Slice image

- Corresponding G-code trajectory

The G-code is converted into sequences of

(X, Y, E)

where

- X = nozzle X-position

- Y = nozzle Y-position

- E = extrusion amount

Each printing layer becomes a supervised training sample.

Stage 2 — Vision Feature Extraction

The input image is processed using a pretrained DinoV2 Vision Transformer (ViT).

Pipeline:

Input Image

↓

Patch Embedding

↓

Vision Transformer

↓

Multi-scale Features

The DinoV2 encoder extracts:

- object boundaries

- geometric structure

- spatial relationships

These features condition the diffusion model.

Stage 3 — Diffusion-Based G-code Generation

A Denoising Diffusion Probabilistic Model (DDPM) generates G-code.

Training:

True G-code

↓

Add Gaussian Noise

↓

Train U-Net

↓

Predict Noise

Inference:

Random Noise

↓

DDPM

↓

Toolpath Sequence

↓

G-code

The diffusion model progressively transforms noise into executable printing trajectories.

Stage 4 — Cross-Attention

The 1D U-Net receives visual information through cross-attention.

Pipeline:

Image Features

↓

Cross Attention

↓

Diffusion U-Net

↓

Toolpath

Thus,

the generated G-code remains consistent with the geometry in the input image.

Stage 5 — Printer Execution

The generated toolpaths are converted directly into

Printer Ready G-code

without

- CAD

- STL

- Slicing software

This significantly reduces manufacturing complexity.

Architecture

Input Image

↓

DinoV2 Vision Encoder

↓

Visual Features

↓

Cross Attention

↓

Diffusion Model (DDPM)

↓

1D U-Net

↓

Toolpath Sequence

↓

G-code

↓

3D Printer

Dataset

The framework uses the Slice-100K dataset, which contains:

- 100,000+ STL–G-code pairs

- Layer-wise slice images

- Corresponding G-code trajectories

Each layer is treated as an independent training sample, improving data efficiency.

Experimental Results

The paper demonstrates that:

- The generated G-code is executable without manual CAD modeling.

- The diffusion model produces coherent nozzle trajectories conditioned on image features.

- The framework reduces workflow complexity by removing CAD and slicing stages.

- The slice-based representation enables efficient training and inference for 2D and 2.5D geometries.

Strengths

1. Eliminates CAD Modeling

Users only provide

- sketch

- photograph

- slice image

No CAD software required.

2. End-to-End Pipeline

Entire workflow:

Image

↓

Printer

3. Diffusion Model

Produces smooth

- nozzle trajectories

- extrusion paths

4. Uses DinoV2

Strong vision representation improves geometric understanding.

5. Large Dataset

100k+ paired samples.

6. Faster Manufacturing

Removes

- CAD

- STL

- Slicing

making rapid prototyping much easier.

Limitations

1. No CAD Model

Output is

G-code

not

Editable CAD

Once printed,

editing becomes difficult.

2. Only Supports 2D and 2.5D Objects

The current framework is designed for geometries that can be represented as one or a small set of layer-wise consistent cross-sections. Complex 3D parts with significant layer-to-layer topological changes are outside its current scope.

3. No Semantic Understanding

Cannot understand

Design a hospital chair.

Only reproduces geometry from images.

4. No Existing CAD Preservation

No CAD history

No feature tree

No parametric editing.

5. No QR Representation

No compressed feature storage.

6. No Context-Aware Design

Cannot redesign

Office Chair

↓

Hospital Chair

It only converts image information into printer instructions.

How Your Method is Better

Your Proposed Pipeline

CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic CAD Model

| Image2Gcode | Your MTP |
| --- | --- |
| Input is image or sketch | Input is an existing CAD model |
| Output is G-code | Output is a new editable CAD model |
| Eliminates CAD | Preserves and extends CAD |
| No semantic reasoning | Uses LLM for semantic understanding |
| No QR representation | QR-based compact feature encoding |
| Manufacturing-focused | Design-generation-focused |
| Cannot modify designs | Can generate application-specific redesigns |
| Supports printing only | Supports editing, redesign, and manufacturing |

Research Gap

Image2Gcode demonstrates that deep learning can directly generate printer-ready G-code from images, removing the need for CAD modeling and slicing. However, it shifts the focus from design to manufacturing, producing machine instructions rather than editable CAD models. It also lacks semantic reasoning, feature preservation, and compact design representations. Your MTP addresses these limitations by retaining the CAD model as the primary output, encoding its geometric features into a QR code, and using LLM-guided natural language prompts to create context-aware synthetic CAD models that remain editable and suitable for downstream engineering workflows.

Overall Summary

Image2Gcode proposes an end-to-end manufacturing pipeline that generates printer-ready G-code directly from images using a DinoV2 vision encoder, a DDPM-based diffusion model, and a 1D U-Net with cross-attention. By eliminating CAD modeling and slicing, it simplifies additive manufacturing and accelerates rapid prototyping for suitable geometries. However, its output is manufacturing instructions rather than editable CAD, making it unsuitable for iterative design modification or semantic adaptation. Your proposed MTP extends beyond manufacturing by combining QR-based CAD feature encoding with LLM-driven semantic reasoning, enabling the reconstruction, modification, and generation of context-aware, editable CAD models instead of printer-specific toolpaths.

Paper 9

Paper Name

Generative AI as a Tool for Transforming Digital Information into 3D Geometry

Authors: Cosmin Gabriel Grădinaru, Nicolae Răzvan Mititelu, Oussama Abda, Irina Ignătescu-Manea, Liviu Pruna

Journal: Journal of Industrial Design and Engineering Graphics (2026)

Problem Statement

Large Language Models (LLMs) like ChatGPT can generate Python/OpenSCAD code to create 3D models. However, directly generating 3D printable QR code models presents several challenges:

- Inconsistent AI outputs

- Topological errors

- Geometry defects

- Material separation issues

- Incompatibility with slicing software (e.g., Bambu Studio)

The authors aim to develop a reliable AI-assisted workflow that consistently generates 3D-printable QR code models for additive manufacturing.

Core Idea

Instead of relying on ChatGPT to directly generate a valid 3D model, the authors introduce an iterative prompt refinement and validation process.

The workflow is:

Digital Information

↓

ChatGPT

↓

Python Code

↓

OBJ File

↓

Validation

↓

Prompt Refinement

↓

Final Printable QR Model

The major contribution is not a new AI model, but a systematic methodology that improves the reliability of AI-generated 3D QR codes.

Logic Behind the Principle

The proposed approach consists of five stages.

Stage 1 — QR Code Generation

A digital link is converted into a standard QR code.

Example:

Website URL

↓

QR Matrix

The QR matrix follows the ISO/IEC 18004 standard to ensure it remains scannable after 3D printing.

Stage 2 — AI Code Generation

The user prompts ChatGPT to generate Python code that creates a 3D QR model.

The generated code produces:

- White base plate

- Black QR modules

These are exported as an OBJ file for 3D printing.

Stage 3 — Validation

Every generated model is evaluated using three criteria:

- Geometric Integrity

  - Is the model topologically valid?

- QR Scannability

  - Can the printed QR code be scanned?

- Slicer Compatibility

  - Can Bambu Studio correctly import and process the OBJ file?

Only models satisfying all three conditions are accepted.

Stage 4 — Prompt Optimization

When failures occur, prompts are refined.

Typical corrections include:

- Better geometric constraints

- Proper module dimensions

- Correct alignment

- Material separation

This iterative process significantly improves generation quality.

Stage 5 — Universal Python Script

After multiple iterations, the authors produce a general-purpose Python program that:

- Generates QR matrices

- Creates the white base

- Builds black QR modules

- Combines them into a multi-material OBJ model

The resulting file is compatible with modern slicers and suitable for multi-material 3D printing.

Architecture

Digital Information

↓

QR Matrix

↓

ChatGPT

↓

Python Code

↓

OBJ Model

↓

Validation

↓

Prompt Refinement

↓

Optimized Python Script

↓

Printable QR Model

Methodology

The workflow is evaluated in a controlled environment using ChatGPT 5.1.

Each generated model undergoes:

- Visual CAD inspection

- QR scanning tests

- Bambu Studio import

- Multi-material slicing validation

This ensures the generated models are both printable and functional.

Experimental Results

The paper reports:

| Before Optimization | After Optimization |
| --- | --- |
| ~40–50% successful models | >90% successful models |
| Frequent topology errors | Stable geometry |
| QR often unreadable | Fully scannable QR codes |
| Frequent slicing failures | Compatible with Bambu Studio |

These improvements result from iterative prompt engineering and validation rather than changes to the underlying LLM.

Strengths

1. Practical AI Workflow

Demonstrates how LLMs can assist engineering design through structured prompt refinement.

2. High Reliability

Optimization improves the success rate to over 90%.

3. Multi-material Support

Generates QR models with separate printable components.

4. Fully Reproducible

Provides an optimized Python workflow rather than relying on one-off AI outputs.

5. Manufacturing-Oriented

Focuses on producing models that are directly usable in additive manufacturing.

6. Demonstrates Importance of Human-in-the-Loop

Shows that combining AI with validation and human expertise produces more reliable engineering results.

Limitations

1. Not a CAD Reconstruction Method

The paper only generates QR code models, not general CAD objects.

2. No Feature Encoding

The QR code simply stores digital information.

It does not encode geometric or semantic features of a CAD model.

3. No AI-Based CAD Generation

The generated output is

QR

↓

OBJ

There is no reconstruction or generation of new CAD designs.

4. No Natural Language Design Modification

The workflow cannot answer:

Convert this chair into a hospital chair.

5. Domain Specific

The approach is limited to 3D printable QR codes.

It is not applicable to

- CAD reconstruction

- Reverse engineering

- Mechanical design generation

6. Manual Validation Required

The iterative refinement process depends on repeated human evaluation and correction.

How Your Method is Better

Your Proposed Pipeline

Existing CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic CAD Model

| This Paper | Your MTP |
| --- | --- |
| QR stores digital information | QR stores geometric and structural CAD features |
| Generates only QR geometry | Generates complete synthetic CAD models |
| Manufacturing-focused | CAD design-focused |
| No semantic understanding | Uses LLMs for semantic reasoning |
| No reconstruction | Reconstructs and redesigns CAD models |
| Static QR representation | Dynamic QR-guided CAD generation |
| No engineering adaptation | Supports context-aware redesign (e.g., hospital chair) |

Research Gap

This paper demonstrates how generative AI can reliably create 3D-printable QR codes through prompt refinement and validation. However, the QR code is treated only as a carrier of digital information. It is not used as a compact representation of CAD geometry or semantic features. Furthermore, the workflow neither reconstructs nor generates CAD models from QR data. Your MTP addresses these limitations by transforming the QR code into a feature descriptor for CAD models, enabling LLM-guided reconstruction and context-aware synthetic CAD generation from the encoded information.

Overall Summary

This paper focuses on using generative AI to automate the creation of 3D-printable QR codes for additive manufacturing. Its key contribution is an iterative workflow that combines LLM-generated Python code, prompt refinement, and validation to achieve highly reliable QR models compatible with 3D printing. While valuable for demonstrating AI-assisted engineering workflows, it is not a CAD generation or reconstruction framework. For your MTP, its main relevance is the concept of using QR codes as compact digital carriers. Your work extends this idea significantly by using QR codes to encode CAD geometry and structural features, allowing LLMs to reconstruct, modify, and generate new context-aware CAD models from the encoded representation.

Paper 10

Paper Name

Req2CAD: Bridging Functional Requirements and Parametric CAD Models to Support Conceptual 3D Design

Authors: Qianzhi Jing, Hankai Lu, Shuojin Huang, Peter Childs, Liuqing Chen

Conference: CHI 2026 (ACM Conference on Human Factors in Computing Systems)

Problem Statement

Current CAD generation systems have several major limitations:

- Most AI methods reconstruct existing CAD models instead of designing new ones.

- They require detailed inputs such as:

  - Images

  - Point clouds

  - Orthographic views

  - Existing CAD models

- Designers, especially beginners, struggle to convert functional requirements into CAD models.

- Existing Text-to-CAD systems require highly detailed prompts, which are difficult to provide during the conceptual design stage.

The authors aim to bridge the gap between functional requirements (e.g., "Design a mobile phone holder") and editable parametric CAD models by creating an AI-assisted conceptual design system.

Core Idea

Instead of directly asking an LLM to generate CAD,

Req2CAD follows the actual engineering design process.

Design Problem

↓

Function Decomposition

↓

Function → Structure Reasoning

↓

Retrieve Similar CAD Components

↓

Progressive CAD Generation

↓

Editable CAD Model

The system combines

- Functional reasoning

- Knowledge retrieval

- Human interaction

- LLM-based CAD generation

into a single workflow.

Logic Behind the Principle

The framework consists of five stages.

Stage 1 — Functional Decomposition

Input

Design a Mobile Phone Holder

GPT-4o decomposes it into basic functions:

Support phone

↓

Adjust angle

↓

Prevent slipping

↓

Provide stability

Each function follows the verb + object format.

Example:

- Support object

- Store energy

- Reduce vibration

- Hold component

This creates a functional hierarchy before any CAD generation begins.

Stage 2 — Function-to-Structure Reasoning

This is the biggest innovation of the paper.

Instead of generating CAD immediately,

the system searches a Function–Structure Knowledge Base.

The knowledge base contains:

- 128,873 CAD components

- Functional annotations

- Multi-view images

- CADQuery prompts

- B-rep models

Example:

Function

↓

Support Object

↓

CAD Components

↓

Brackets

↓

Arms

↓

Supports

The designer explores candidate solutions before choosing one.

Stage 3 — Design Space Exploration

The retrieved CAD components are organized using two complementary feature representations.

(A) Topological Features

Each CAD model is converted into a B-rep graph.

CAD

↓

B-rep

↓

Graph

↓

WL Kernel Similarity

This compares structural similarity between CAD models.

(B) Geometric Features

Each CAD model is converted into

Mesh

↓

Point Cloud

↓

3DShape2Vec

↓

Embedding

These embeddings represent geometric similarity.

The system visualizes similar designs using t-SNE clustering, allowing designers to browse alternative solutions.

Stage 4 — Progressive CAD Generation

Unlike previous papers,

Req2CAD does not generate the entire CAD model at once.

Example:

Step 1

Generate Cube

↓

CAD

Step 2

User selects one face.

Selected Face

↓

Cut Rectangle

↓

Updated CAD

Step 3

Select another face.

Add Cylinder

↓

Updated CAD

Every iteration extends the previous CADQuery code.

Instead of

Generate Whole CAD

the system performs

Generate

↓

Modify

↓

Modify

↓

Modify

This gives much better control over the final design.

Stage 5 — CAD Editing and Fusion

The system also supports editing existing CAD components.

Capabilities include:

- Sketch editing

- Extrusion modification

- Parameter editing

- Component fusion

To identify fusion opportunities,

Req2CAD detects common geometric regions between two CAD models.

CAD A

CAD B

↓

Feature Matching

↓

Fusion Anchors

↓

New CAD

These "fusion anchors" help designers combine components in meaningful ways.

Architecture

Design Problem

↓

LLM (Function Decomposition)

↓

Function Tree

↓

Knowledge Base Retrieval

↓

CAD Component Recommendation

↓

Topology + Geometry Analysis

↓

Progressive CAD Generation

↓

CAD Editor

↓

Editable CAD Model

Dataset

Req2CAD builds upon the Text2CAD dataset and enriches it with functional information.

The resulting Function–Structure Knowledge Base contains:

- 128,873 CAD components

- Multi-view renderings

- B-reps

- CADQuery prompts

- Functional descriptions

- Function keywords

These annotations were automatically generated using:

- Qwen2.5-VL-7B for function descriptions

- Qwen3-14B for extracting function keywords

- Qwen3-Embedding-4B for retrieval embeddings

Experimental Results

Function Annotation

Automatic annotation achieved approximately:

- 72–74% accuracy

while dramatically reducing manual labeling effort.

CAD Generation

Compared with Minimal JSON, CADQuery generation achieved:

- Lower Point Cloud Distance

- Lower Hausdorff Distance

- Comparable compile rate (~89–90%)

Progressive CAD Generation

Compared with one-step generation:

| Method | Point Cloud Distance | Compile Rate |
| --- | --- | --- |
| One-step | 0.197 | 90% |
| Progressive | 0.013 | 100% |

Progressive generation significantly improved CAD quality and execution reliability.

Strengths

1. Function-Driven CAD Design

Starts from

design requirements

instead of geometry.

2. Human-AI Collaboration

The designer remains involved throughout the process.

3. Huge Knowledge Base

Nearly

129k CAD components

for inspiration.

4. Progressive Generation

Much easier than generating complete CAD in one prompt.

5. Multi-modal Interaction

Supports:

- Text

- Graphical face selection

- CAD editing

- CAD fusion

6. Design Space Exploration

Uses both

- topology

- geometry

to recommend diverse CAD solutions.

7. Produces Editable CAD

Output is

CADQuery

↓

STEP

↓

Onshape

Limitations

1. No QR Representation

The system does not use a compact feature representation such as QR codes for storing or sharing CAD information.

2. Depends on Existing CAD Knowledge Base

The recommendations come from a database of previously annotated CAD components. It cannot infer entirely new structural concepts beyond the knowledge available in the dataset.

3. No Automatic Feature Extraction from CAD

It retrieves existing CAD components but does not automatically extract and encode reusable feature descriptors from arbitrary CAD models.

4. Limited Semantic Redesign

Although it starts from functional requirements, it does not support modifying an existing CAD model with prompts such as:

Convert this office chair into a hospital chair.

5. Component-Level Focus

The workflow is primarily designed for component-level conceptual design. Full product assemblies are created by exporting components to external CAD software (e.g., Onshape) for assembly.

6. Requires Human Interaction

The system is intentionally interactive and depends on designer input at multiple stages, making it less suitable for fully automated CAD generation.

How Your Method is Better

Your Proposed Pipeline

Existing CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic CAD Model

| Req2CAD | Your MTP |
| --- | --- |
| Starts from functional requirements | Starts from an existing CAD model |
| Retrieves similar CAD components | Extracts reusable geometric features directly from the CAD |
| Uses a large CAD knowledge base | Uses QR as a compact feature descriptor |
| Progressive CAD generation | Feature-preserving CAD reconstruction and redesign |
| Human-guided component generation | LLM-guided context-aware CAD transformation |
| No QR representation | QR-based storage and transfer of CAD features |
| Creates conceptual CAD | Creates synthetic variants while preserving the original design |

Research Gap

Req2CAD effectively bridges functional requirements and conceptual CAD generation through function decomposition, knowledge retrieval, and progressive CAD generation. However, it relies heavily on a large annotated CAD knowledge base and does not provide a mechanism to compress, transfer, or reuse the geometric features of an existing CAD model. It also lacks support for semantic redesign of existing products.

Your MTP fills these gaps by introducing QR-based feature encoding of CAD models. Instead of retrieving components from a knowledge base, your approach extracts geometric features through 3D slicing, stores them as a compact QR representation, and combines them with natural language prompts to generate context-aware synthetic CAD models. This enables lightweight sharing, preservation of original design characteristics, and intelligent redesign without requiring a large retrieval database.

Overall Summary

Req2CAD is one of the most comprehensive frameworks for AI-assisted conceptual CAD design. Rather than directly generating CAD from text, it mimics the engineering design process through functional decomposition, function-to-structure reasoning, knowledge-base retrieval, design space exploration, and progressive CAD generation. Its emphasis on human–AI collaboration makes it particularly suitable for early-stage conceptual design. However, it still depends on a large CAD repository and lacks mechanisms for compact CAD feature encoding or semantic redesign of existing models. Your MTP extends this direction by introducing QR-based CAD feature representation and LLM-guided feature-preserving redesign, enabling a novel pipeline that bridges CAD → QR → LLM → Synthetic CAD while maintaining editability and supporting application-specific transformations.

| Method | CAD Reconstruction | CAD Generation | Uses Existing CAD | LLM / AI | NLP Input | QR-based Feature Encoding | Feature Preservation | Context-aware Redesign | Editable CAD Output | Synthetic CAD Generation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DeepCAD (2021) | Yes | Yes | No | No | No | No | No | No | Yes | Yes |
| Ortho2CAD (2026) | Yes | No | No | Yes | No | No | No | No | Yes | No |
| GenCAD-3D (2025) | Yes | Yes | No | No | No | No | No | No | Yes | Yes |
| Text-to-CadQuery (2025) | No | Yes | No | Yes | Yes | No | No | No | Yes | Yes |
| GaussianCAD (2025) | Yes | No | No | No | No | No | No | No | No | No |
| Text2CSG (2026) | No | Yes | No | Yes | Yes | No | No | No | Yes | Yes |
| Image2Gcode (2026) | No | No | No | Yes | No | No | No | No | No | No |
| Generative AI for 3D QR Geometry (2026) | No | No | No | Yes | Yes | Yes (QR only) | No | No | No | No |
| Req2CAD (2026) | Yes | Yes | Partial | Yes | Yes | No | Partial | Partial | Yes | Yes |
| My framework (QRFusionCAD) | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

MINE IDEA-

Absolutely. Based on everything we've discussed, here's your proposed work written in the same format as the research papers.

Proposed Work

Paper Name

QRFusionCAD: Context-Aware Synthetic CAD Generation using QR-Encoded Feature Representation and Large Language Models

(You can change the title later.)

Problem Statement

Existing CAD generation and reconstruction methods suffer from several limitations:

- They generate CAD models from scratch without utilizing existing CAD knowledge.

- They do not preserve the geometric identity of an existing CAD model.

- They require large CAD datasets, images, point clouds, or orthographic drawings.

- Existing QR-based approaches store only digital information rather than CAD features.

- Current systems cannot intelligently redesign an existing CAD model based on user requirements.

The objective of this work is to develop a framework that extracts geometric and structural features from an existing CAD model, compresses them into a QR code, and uses Large Language Models (LLMs) to generate context-aware synthetic CAD models from natural language prompts.

Core Idea

Instead of generating CAD directly from text or images, the proposed framework introduces QR as an intermediate CAD representation.

Existing CAD Model

↓

3D Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

LLM

↓

Synthetic Editable CAD Model

The QR code acts as a lightweight representation of the original CAD model while preserving essential geometric information.

Logic Behind the Principle

Step 1 — CAD Slicing

The input CAD model is divided into multiple cross-sectional slices.

Example:

CAD Chair

↓

Horizontal Slices

↓

Layer-wise Geometry

The slicing process captures the internal and external geometry of the object.

Step 2 — Feature Extraction

For each slice, important geometric and structural features are extracted.

Examples include:

- Boundary contours

- Shape descriptors

- Dimensions

- Topological relationships

- Structural characteristics

These features represent the identity of the CAD model.

Step 3 — QR Encoding

The extracted features are compressed into a QR representation.

CAD Features

↓

Encoding

↓

QR Code

Unlike traditional QR codes that store text or URLs, this QR stores geometric feature information.

Step 4 — Natural Language Interaction

The user provides a design prompt.

Example:

"Convert this office chair into a hospital chair."

The prompt specifies the desired application or modification.

Step 5 — LLM-Based Reasoning

The Large Language Model combines:

- QR-encoded CAD features

- User intent

to understand:

- what should remain unchanged,

- what needs to be modified,

- how the new CAD model should satisfy the requested context.

Step 6 — Synthetic CAD Generation

Finally,

the system generates a new editable CAD model.

Example:

Office Chair

↓

QR Features

+

Hospital Chair Prompt

↓

Synthetic Hospital Chair CAD

The generated model preserves the important characteristics of the original chair while adapting it to the new application.

Proposed Architecture

Existing CAD Model

↓

CAD Slicing

↓

Feature Extraction

↓

QR Encoding

↓

Natural Language Prompt

↓

Large Language Model

↓

Feature-aware CAD Generator

↓

Synthetic Editable CAD Model

Expected Contributions

- Develop a QR-based representation for CAD features.

- Enable compact storage and transfer of CAD information.

- Preserve important geometric characteristics of existing CAD models.

- Introduce LLM-based semantic reasoning for CAD redesign.

- Generate context-aware synthetic CAD models from existing designs.

- Reduce dependence on large CAD retrieval databases.

Expected Advantages

- Lightweight CAD representation

- Easy sharing through QR codes

- Feature-preserving CAD reconstruction

- Natural language interaction

- Context-aware design modification

- Editable CAD output

- Reduced storage requirements

- Improved reusability of CAD models

Possible Applications

- Smart manufacturing

- Reverse engineering

- Product customization

- Furniture design

- Healthcare equipment design

- Automotive components

- Aerospace component redesign

- Digital twins

- Industry 4.0

Scenario 1 — Physical Product (This is the strongest use case)

Suppose a hospital has 10,000 chairs.

Every chair has a QR sticker.

Hospital Chair

↓

QR Sticker

The QR contains

Model ID : HC-1023

Feature ID : F2345

Manufacturer

Material

Version

An engineer scans it.

Scan QR

↓

Retrieve CAD Features

↓

LLM

↓

"Make it suitable for children"

↓

New CAD

Here the QR connects the physical object to its digital design.

Scenario 2 — Cloud Retrieval

Instead of storing CAD locally,

QR stores

Feature ID

Cloud Link

Version

When scanned,

QR

↓

Database

↓

CAD Features

↓

LLM

Now QR becomes a key.

Scenario 3 — Manufacturing

Factory

Part

↓

QR

↓

Scan

↓

Open CAD

↓

Modify

↓

Manufacture

Scenario 4 — Spare Parts

Imagine a broken chair.

Instead of searching folders,

Scan QR

↓

Retrieve CAD

↓

Modify

↓

Print