# Paper Review — Paper 1

## DeepCAD: A Deep Generative Network for Computer-Aided Design Models

Authors: Rundi Wu, Chang Xiao, Changxi Zheng

Conference: ICCV 2021

Institution: Columbia University

# Problem Statement

Most existing deep learning models generate 3D objects as point clouds, meshes, or voxels. These representations are suitable for visualization but cannot be directly edited in CAD software, making them impractical for engineering and manufacturing applications.

The authors aim to generate editable CAD models instead of discrete 3D representations.

# Core Idea

Instead of generating a mesh, the model generates the sequence of CAD operations that a designer would perform, e.g. Sketch Circle → Extrude → Boolean Union → Final CAD Model.

The CAD model is treated similarly to a sentence in natural language, where each CAD command is considered a "word." The authors employ a Transformer-based Autoencoder to learn CAD command sequences.

# Logic Behind the Principle

## Step 1 — CAD Representation

A CAD object is converted into commands such as Sketch, Line, Arc, Circle, Extrude, and Boolean. Every command has parameters (e.g. Line: (x,y); Circle: (center, radius); Extrude: (distance, direction)). Instead of learning geometry directly, the model learns CAD commands plus command parameters. This is easier because CAD software already represents objects this way.

## Step 2 — Convert Commands into Neural Input

Neural networks require fixed-size vectors. The authors convert every command into a command type plus a 16-dimensional parameter vector. Continuous values are quantized into 256 discrete levels because regression introduces small numerical errors that can destroy CAD constraints (parallel edges, perpendicular faces, etc.). Classification into discrete bins preserves geometric relationships better.

## Step 3 — Transformer Encoder

Each command is embedded using its command type, parameters, and position in sequence. The Transformer Encoder produces a 256-dimensional latent vector, which becomes the compressed representation of the CAD model.

## Step 4 — Transformer Decoder

The decoder receives the latent vector, passes it through a Transformer, and reconstructs CAD commands, which form the CAD model. Thus the network reconstructs the original CAD operations.

## Step 5 — Random CAD Generation

After training, the authors train a GAN on the latent vectors. Random noise is passed through the GAN to produce a latent vector, which the Transformer Decoder converts into a new CAD model. Thus the system can generate completely new CAD models.

# Dataset

Existing CAD datasets were too small. The authors built a dataset containing 178,238 CAD models collected from Onshape. Each model contains the full CAD construction sequence. This is one of the major contributions of the paper.

# Experimental Results

- 99.5% command accuracy

- 97.98% parameter accuracy

- Low invalid CAD generation rate (~2.7%)

- Generated CAD models are editable in commercial CAD software

# Strengths

- First deep model to generate editable CAD programs — output can directly open inside CAD software, unlike meshes

- Transformer captures sequential CAD operations — CAD modeling is naturally sequential and the Transformer fits this representation well

- Large dataset — 178k CAD models significantly outperform previous CAD datasets in scale

- High reconstruction accuracy — very low reconstruction error

- Editable output — generated models remain editable, which is extremely valuable in engineering

# Limitations

## 1. Limited CAD Operations

Supports only Line, Arc, Circle, Extrude, and Boolean. No support for Fillet, Chamfer, Loft, Sweep, Shell, Revolve, or complex free-form surfaces.

## 2. No Semantic Understanding

The model only understands geometry. It cannot answer a request such as "Convert this chair into a hospital chair." There is no understanding of object function or context.

## 3. No Natural Language Interface

Input is always CAD commands, not a text prompt.

## 4. No Feature Compression

The entire CAD command sequence must be stored. There is no compact representation like a QR code.

## 5. Cannot Modify Existing Objects Based on Context

It only reconstructs or randomly generates. There is no intelligent redesign.

## 6. Topological Errors Still Exist

Some generated CAD sequences become invalid. The paper acknowledges this limitation.

# How Your Method is Better

Your Proposed Pipeline: CAD Model → 3D Slicing → Feature Extraction → QR Encoding → User Prompt → LLM / Generative AI → Synthetic CAD Model.

| DeepCAD | Your Method |
| --- | --- |
| Generates CAD command sequences | Generates context-aware synthetic CAD models |
| Uses Transformer only | Uses LLM/Generative AI for semantic reasoning |
| Requires full CAD sequence | Stores compressed geometric features in a QR code |
| No semantic understanding | Understands user intent (e.g., "hospital chair") |
| No natural language support | Supports natural language queries |
| Only reconstructs or generates random CAD | Reconstructs and adapts CAD models based on context |
| Geometry only | Combines geometry with semantic information |
| No portable encoding | QR acts as a lightweight, shareable representation |

# Research Gap

This paper focuses on how to generate CAD programs, but it does not address: efficient storage or transfer of CAD knowledge, semantic feature encoding, natural language–guided design modification, or context-aware CAD synthesis.

Your MTP proposes to bridge these gaps by introducing a QR-encoded feature representation and using generative AI to reconstruct and adapt CAD models according to user requirements.

# Overall Summary

DeepCAD is a foundational paper because it demonstrates that CAD models can be represented as sequences of operations and generated using Transformer architectures. Its main contribution is moving from mesh-based generation to editable CAD generation. However, it is limited to geometric reconstruction and lacks semantic reasoning, natural language interaction, and compact feature encoding. Your proposed MTP extends this direction by combining CAD slicing, QR-based feature encoding, and generative AI to enable context-aware, natural language–driven synthetic CAD reconstruction.

# Paper Review — Paper 2

## ORTHO2CAD: 3D CAD Generation from Orthographic Drawings using Vision Language Models

Authors: Aditya Joglekar, Amit Regmi, Kenji Shimada, Levent Burak Kara

Institution: Carnegie Mellon University (2026)

# Problem Statement

In industry, engineers usually exchange 2D orthographic drawings rather than editable CAD models because drawings are easier to share and protect intellectual property. However, manufacturing, simulation, and design modification require editable 3D CAD models. Currently, engineers manually recreate the CAD model from these drawings, which is time-consuming.

The paper aims to automatically reconstruct an editable CAD model from rasterized orthographic drawings.

# Core Idea

The paper converts an orthographic drawing, via a Vision Language Model (Qwen3-VL), into CadQuery code, which is executed by a Python interpreter to produce an editable CAD model.

Instead of predicting meshes or point clouds, the model predicts CadQuery code, which can be executed to generate a STEP/CAD model. It combines Vision-Language Models (VLMs), Supervised Fine-Tuning (SFT), and Reinforcement Learning (RL) to improve CAD reconstruction.

# Logic Behind the Principle

## Step 1 — Generate Orthographic Drawings

Since no large dataset exists, the authors generate synthetic engineering drawings from STEP files. Each drawing contains a front view, top view, side view, hidden lines, and basic dimensions, using pythonOCC.

## Step 2 — Vision Language Model

The input is an orthographic drawing plus a prompt (e.g. "Generate the CadQuery code needed to create the CAD"), fed into Qwen3-VL. The VLM learns the relationship between drawing features and CAD commands.

## Step 3 — Supervised Fine-Tuning (SFT)

For datasets that already contain CadQuery code (DeepCAD), the model is trained using the drawing and ground-truth CadQuery code with a cross-entropy loss. The objective is to predict the exact CAD program.

## Step 4 — Reinforcement Learning

Most CAD datasets don't contain CadQuery code. Instead of comparing code, they compare geometry: the drawing produces generated CadQuery code, which is executed into a 3D model and compared via IoU with the ground truth to produce a reward. Higher IoU gives a higher reward and lower IoU gives a lower reward. This allows learning without ground-truth code.

## Step 5 — Final CAD Reconstruction

The generated CadQuery program is executed to produce an editable STEP model, which can be modified in CAD software.

# Dataset

The authors generate 150,000+ orthographic drawings using DeepCAD and Fusion360. Each drawing contains three orthographic views, hidden lines, and bounding-box dimensions. This is one of the major contributions of the paper.

# Experimental Results

## DeepCAD Dataset

- 100% valid CadQuery code

- IoU = 0.7922

- 7.6% improvement over CAD-Coder

## Fusion360 Dataset

After RL:

- 100% executable code

- IoU = 0.5601

- Better than GPT-5.2 and CAD-Coder baselines

# Strengths

- Uses real engineering drawings — front, side, and top views, aligning with industrial workflows, unlike previous works using perspective images

- Produces editable CAD — output is CadQuery code, not meshes

- Reinforcement learning — can learn even without ground-truth CAD code, which is very practical

- Large dataset — 150k+ orthographic drawings

- High CAD validity — 100% executable code

# Limitations

## 1. Requires Complete Orthographic Drawings

The system depends on the front view, side view, and top view. If any view is missing, performance drops.

## 2. No Semantic Understanding

The model only reconstructs geometry. It cannot understand a request such as "Convert this chair into a hospital chair."

## 3. No Design Adaptation

Input to output is almost one-to-one. It reconstructs; it does not redesign.

## 4. No Compact Representation

The whole drawing must be supplied every time. There is no compressed representation such as QR.

## 5. Limited CAD Operations

The dataset mainly contains Sketch and Extrude. Complex operations like Fillet, Loft, Shell, and Sweep are mostly absent.

## 6. Still Struggles with Thin Structures

The paper itself shows thin plates, thin rods, and hollow structures often have low IoU.

# How Your Method is Better

Your Proposed Pipeline: CAD Model → 3D Slicing → Feature Extraction → QR Encoding → User Prompt ("Hospital Chair") → LLM → Synthetic CAD Model.

| Ortho2CAD | Your Method |
| --- | --- |
| Requires three orthographic drawings | Requires only a QR code containing extracted features |
| Reconstructs existing CAD | Reconstructs and redesigns CAD |
| No semantic understanding | Uses natural language prompts to understand design intent |
| Geometry only | Combines geometry + semantics |
| Needs complete drawings every time | Uses a compact QR representation for storage and transfer |
| Produces one reconstruction | Generates multiple context-aware synthetic CAD variants |
| No object adaptation | Can modify the object for new domains (hospital, office, classroom, etc.) |
| Dependent on orthographic drawings | Works from QR-encoded features plus user intent |

# Research Gap

Although Ortho2CAD successfully reconstructs editable CAD models from orthographic drawings, it still treats reconstruction as a geometry translation task. It does not encode object semantics, compress design knowledge into a portable representation, or support context-aware redesign through natural language. Your MTP addresses these gaps by introducing QR-based feature encoding and combining it with LLMs to generate synthetic, context-aware CAD models (e.g., adapting a generic chair into a hospital chair while preserving essential structural characteristics).

# Overall Summary

Ortho2CAD is one of the first works to reconstruct editable CAD models directly from engineering orthographic drawings using Vision-Language Models, supervised fine-tuning, and reinforcement learning. Its major contribution is bridging 2D engineering drawings and executable CadQuery programs. However, it remains focused on reconstructing existing geometry and does not perform semantic design transformation or compact feature encoding. Your proposed MTP extends this direction by representing CAD features in a QR code and leveraging generative AI to create new, context-aware CAD designs from user intent while preserving the core geometry.

# Paper Review — Paper 3

## GenCAD-3D: CAD Program Generation using Multimodal Latent Space Alignment and Synthetic Dataset Balancing

Authors: Nomi Yu, Md Ferdous Alam, A. John Hart, Faez Ahmed

Institution: Massachusetts Institute of Technology (MIT)

Journal: Journal of Mechanical Design (2025 Preprint)

# Problem Statement

Modern AI models can generate meshes, point clouds, and 3D shapes, but engineers require editable CAD programs rather than static geometry. Reverse engineering currently involves scanning a physical object in 3D and then manually reconstructing the CAD model, which is time consuming, requires CAD experts, and is difficult for complex geometries.

Another challenge is that existing datasets (DeepCAD) contain mostly simple CAD models, causing AI models to perform poorly on complex parts.

# Core Idea

Instead of directly generating CAD from geometry, GenCAD-3D first learns a shared latent space between CAD programs, point clouds, and meshes using contrastive learning. It then uses a latent diffusion model to convert geometric representations into CAD programs.

Finally, the authors introduce SynthBal, a synthetic dataset balancing strategy that improves learning on complex CAD programs.

# Logic Behind the Principle

The complete pipeline consists of three stages.

## Stage 1 — Learn CAD Representation

Every CAD model is represented as a sequence of Sketch and Extrude operations, each containing a command and parameters (e.g. Circle: radius, center; Extrude: distance). These commands are converted into a 60 × 17 matrix, where the first column is the command and the remaining 16 columns are parameters. A Transformer Autoencoder compresses this into a latent vector representing the CAD model.

## Stage 2 — Multimodal Latent Space Alignment

This is the novelty of the paper. Instead of only learning CAD, the model learns a common embedding space for meshes, point clouds, and CAD programs using contrastive learning. If a mesh and a CAD program represent the same object (e.g. the same chair), their latent vectors should be close; if they represent different objects (e.g. a chair versus a table), their latent vectors should be far apart. Positive pairs are pulled together and negative pairs pushed apart using a cosine similarity loss.

## Stage 3 — Conditional Latent Diffusion

After learning the shared latent space, the model trains a diffusion model: a mesh is encoded into a mesh latent, diffusion converts this into a CAD latent, and a Transformer Decoder reconstructs the CAD program. Instead of predicting CAD directly, it predicts the CAD latent vector, which is easier to learn.

## Stage 4 — SynthBal Dataset

The existing DeepCAD dataset (178k CAD models) contains mostly simple CAD; complex CAD models are rare. Therefore, the authors generate synthetic CAD programs using two augmentation methods: Noise, which adds small perturbations to CAD parameters (e.g. a radius of 20 becomes 21), and Replace Sketch, which randomly replaces sketch operations with sketches from another CAD model (e.g. swapping a chair seat for a different chair seat). Only valid CAD models are kept. The final balanced dataset contains 170k samples or 1 million samples depending on training configuration.

# Architecture

Mesh → Mesh Encoder → Mesh Latent Vector → Contrastive Alignment → Shared Latent Space → Conditional Diffusion → CAD Latent Vector → Transformer Decoder → CAD Program → Editable CAD Model.

# Dataset

The paper extends DeepCAD into multiple modalities. Training data includes 168,656 valid CAD programs, a mesh dataset, a point cloud dataset, and a point cloud + normals dataset. Additionally, the authors generate SynthBal (170k) and SynthBal-1M (1 million) synthetic datasets for balancing complex CAD sequences.

# Experimental Results

## Autoencoder Performance

The SynthBal-1M FT model achieved:

- Command Accuracy: 99.66%

- Parameter Accuracy: 98.61%

- IoU: 96.5%

- Invalid CAD Ratio: 0.807%

## Retrieval Performance

Using mesh embeddings, Top-1 retrieval accuracy:

- 99.7% (library size = 10)

- 97.2% (128)

- 87.9% (1024)

- 81.6% (2048)

## Conditional CAD Generation

Best model:

- Command Accuracy: 60.48%

- Parameter Accuracy: 52.64%

- Better reconstruction using mesh than point clouds

# Strengths

- First multimodal latent alignment — aligns mesh, point cloud, and CAD inside one embedding space

- Better reverse engineering — supports going from a 3D scan to editable CAD instead of manual reconstruction

- SynthBal — a novel synthetic dataset balancing strategy that significantly improves learning on complex CAD programs

- Diffusion-based CAD generation — uses latent diffusion instead of directly predicting CAD commands, providing more robust conditional generation

- Supports multiple input modalities — mesh, point cloud, and point cloud + normals

- Large-scale dataset — 168k CAD models and 1 million synthetic CAD samples

# Limitations

## 1. No Natural Language Understanding

Input must be a mesh or point cloud. It cannot understand a request such as "Convert this chair into a hospital chair."

## 2. No Semantic Information

The model only learns geometry. It has no knowledge about hospital, office, classroom, or ergonomic design context.

## 3. QR Encoding Not Used

The entire latent representation must be stored. There is no lightweight transferable representation.

## 4. Limited CAD Commands

Although the architecture can be extended, experiments mainly use Line, Arc, Circle, and Extrude. Complex industrial operations such as fillet, sweep, loft, revolve, and shell are not included in the experiments.

## 5. Moderate Reconstruction Accuracy

Even the best conditional model achieves around 60% command accuracy and 53% parameter accuracy, showing that geometry-to-CAD reconstruction remains challenging.

## 6. High Computational Cost

The framework trains a Transformer Autoencoder, a Contrastive Encoder, and a Diffusion Model, and uses H100 GPUs with large synthetic datasets, making training expensive.

# How Your Method is Better

Your Proposed Pipeline: CAD Model → 3D Slicing → Feature Extraction → QR Encoding → Natural Language Prompt → LLM + Generative AI → Synthetic CAD Model.

| GenCAD-3D | Your MTP |
| --- | --- |
| Requires mesh or point cloud | Uses QR-encoded features from a CAD model |
| Focuses on reverse engineering | Focuses on context-aware CAD redesign and generation |
| No natural language interface | Supports prompts such as "Design this chair for a hospital" |
| Learns only geometric embeddings | Combines geometric features with semantic understanding |
| Stores high-dimensional latent vectors | Uses a compact QR code for storage and sharing |
| Generates CAD similar to the input | Generates modified, application-specific CAD models |
| Limited to reconstruction | Supports reconstruction and intelligent redesign |

# Research Gap

GenCAD-3D demonstrates that multimodal latent alignment can bridge meshes, point clouds, and CAD programs, but it remains a geometry-centric approach. It does not encode design semantics, provide a compact transferable representation, or enable user-driven design adaptation through natural language. Your MTP addresses these gaps by introducing QR-based feature encoding and using LLMs to generate context-aware CAD models that preserve core geometry while adapting to new application domains.

# Overall Summary

GenCAD-3D is a significant advance in automated CAD generation by combining multimodal latent space alignment, contrastive learning, latent diffusion, and synthetic dataset balancing. It improves reverse engineering from meshes and point clouds into editable CAD programs and achieves strong reconstruction and retrieval performance. However, its focus remains on reconstructing geometry rather than understanding design intent. Your proposed MTP extends this direction by integrating QR-encoded geometric features with natural language-driven generative AI, enabling compact representation, semantic reasoning, and application-specific synthetic CAD generation.

# Paper Review — Paper 4

## Text-to-CadQuery: A New Paradigm for CAD Generation with Scalable Large Model Capabilities

Authors: Haoyang Xie, Feng Ju

Institution: Arizona State University

Year: 2025 (Preprint)

# Problem Statement

Current Text-to-CAD methods generate intermediate CAD command sequences instead of directly generating editable CAD models. These command sequences require custom neural architectures, need conversion into CAD vectors or JSON before creating a CAD model, require training models from scratch, and increase computational complexity.

The authors ask why intermediate command sequences should be generated at all, and instead propose generating CadQuery code directly from natural language.

# Core Idea

The paper replaces the traditional CAD generation pipeline (Text → CAD Commands → JSON → CAD Model) with a direct approach: Text → Large Language Model → CadQuery Python Code → Execute → Editable CAD Model.

Since CadQuery is a Python-based CAD scripting language, pretrained LLMs already possess Python coding knowledge, making it easier to fine-tune them for CAD generation.

# Logic Behind the Principle

The paper consists of four major stages.

## Stage 1 — Build a Large Dataset

The authors extend the Text2CAD dataset, which originally contains natural language descriptions and CAD command sequences, by adding CadQuery code for every CAD model. This produces text-prompt-to-CadQuery-code pairs. Approximately 170,000 text–CadQuery pairs are created.

## Stage 2 — Automatic CadQuery Annotation

Instead of manually writing CadQuery code, the authors use Gemini 2.0 Flash. A minimal JSON description is passed to Gemini to produce CadQuery code, which is executed to produce an STL model, rendered as an image, validated by Gemini, and then human-verified. If execution fails, Gemini receives the error message and corrects the code. This iterative refinement improves execution success from 53% to 85% during annotation.

## Stage 3 — Fine-Tune Large Language Models

Six pretrained LLMs are fine-tuned: CodeGPT-small (124M), GPT-2 Medium, GPT-2 Large, Gemma-3-1B, Qwen2.5-3B, and Mistral-7B (LoRA). Instead of training from scratch, they only fine-tune existing models.

## Stage 4 — Generate CAD

Given an input such as "Generate a cylindrical container," the LLM produces CadQuery Python code (e.g. using cq.Workplane to define and extrude a circle), which is executed to produce an editable CAD model. The generated Python code directly creates the CAD model without any intermediate CAD command representation.

# Architecture

Natural Language → Tokenizer → Pretrained LLM → CadQuery Python Code → Python Execution → Editable CAD Model. This architecture is much simpler than previous Text2CAD pipelines because it removes the command-sequence decoding stage.

# Dataset

The dataset is built from DeepCAD and Text2CAD, extended with approximately 170,000 text–CadQuery pairs and 600,000+ natural language prompts. This is currently one of the largest Text-to-CAD datasets using executable CadQuery scripts.

# Experimental Results

The best-performing model (Qwen2.5-3B) achieved:

- 69.3% Top-1 Exact Match

- 48.6% reduction in Chamfer Distance

- 10.5% improvement over the Text2CAD baseline

The Mistral-7B model produced the lowest invalid code rate (1.32%), showing excellent code executability, while Qwen2.5-3B achieved the best overall reconstruction accuracy.

# Strengths

- Direct text-to-CAD generation — no intermediate CAD command representation, resulting in a simpler pipeline

- Uses existing LLMs — no need to design custom Transformer architectures, leveraging pretrained Python knowledge

- Large dataset — 170k paired samples

- Faster training — the smallest model trains in 58 minutes compared to 2 days for Text2CAD

- Better CAD accuracy — produces better CAD reconstruction than Text2CAD across Chamfer Distance, IoU, and F1 metrics

- Scalable — performance improves consistently as model size increases (up to Qwen2.5-3B), demonstrating the benefits of scaling pretrained LLMs for CAD generation

# Limitations

## 1. Only Generates from Text

Input must be a natural language description. It cannot use existing CAD models, QR codes, point clouds, or feature descriptors.

## 2. No Feature Preservation

If a chair already exists, the model does not preserve its geometric identity. Every design is generated solely from text.

## 3. No Context-Based Redesign

The system cannot transform an existing chair into a hospital chair; it only creates CAD from text.

## 4. No Compact Representation

The entire CAD program must be regenerated every time. There is no QR encoding.

## 5. Dependent on Training Data

Performance drops for larger models because 170k samples are still relatively small for 7B models. The authors explicitly mention that larger datasets (13B, 32B models) would likely improve results.

## 6. No Semantic Memory

Generated CAD does not preserve functional purpose, material, or usage domain. It only follows the text prompt.

# How Your Method is Better

Your Proposed Pipeline: Existing CAD Model → 3D Slicing → Feature Extraction → QR Encoding → Natural Language Prompt → LLM → Synthetic CAD Model.

| Text-to-CadQuery | Your MTP |
| --- | --- |
| Input is only natural language | Uses an existing CAD model plus user intent |
| Generates CAD from scratch | Reconstructs and modifies existing designs |
| No geometric feature preservation | Preserves extracted geometric and structural features |
| No compact representation | Encodes features into a lightweight QR code |
| Text-only generation | Combines QR features with natural language |
| No context-aware adaptation | Can redesign objects for new environments (e.g., hospital chair) |
| Produces one CAD model | Can generate multiple synthetic design variants while retaining key characteristics |

# Research Gap

The paper successfully demonstrates that CadQuery is a superior output representation for text-to-CAD generation, replacing task-specific command sequences with executable Python code. However, it assumes that natural language alone is sufficient to describe a design. It does not support reconstructing or modifying an existing CAD model, preserving geometric features, or storing design information in a transferable format. Your MTP addresses these limitations by introducing a QR-based feature representation that captures geometric information from an existing CAD model and combines it with LLM-driven semantic reasoning to generate context-aware synthetic CAD models.

# Overall Summary

Text-to-CadQuery presents a new paradigm for CAD generation by directly translating natural language into executable CadQuery scripts using pretrained LLMs. By eliminating intermediate CAD command sequences, the method simplifies the generation pipeline, reduces training time, and improves reconstruction quality. However, it remains a text-only generative system and lacks mechanisms for preserving or adapting existing designs. Your proposed MTP extends this concept by integrating QR-encoded geometric features with natural language prompts, enabling not only CAD generation but also feature-preserving, context-aware redesign of existing CAD models.

# Paper Review — Paper 5

## GaussianCAD: Robust Self-Supervised CAD Reconstruction from Three Orthographic Views Using 3D Gaussian Splatting

Authors: Zheng Zhou et al.

Journal: Elsevier (Preprint 2025)

Institution: Beijing University of Posts and Telecommunications, China

# Problem Statement

Most CAD reconstruction methods require vector CAD sketches and ground-truth 3D CAD models. Both are difficult to obtain in real industrial applications. Additionally, traditional reconstruction methods fail when sketches contain noise, endpoints shift, or drawings are scanned, because they depend on exact geometric coordinates.

The authors aim to reconstruct 3D CAD models directly from raster orthographic drawings without requiring vector sketches or ground-truth 3D models.

# Core Idea

Instead of treating CAD reconstruction as sketch-to-CAD generation, the paper formulates it as orthographic views → sparse-view 3D reconstruction → 3D CAD. They adapt 3D Gaussian Splatting (3DGS), originally developed for natural-image 3D reconstruction, to work with engineering drawings.

The key innovation is treating CAD reconstruction as a special case of sparse-view reconstruction.

# Logic Behind the Principle

The proposed framework consists of four major stages.

## Stage 1 — Sketch Filtering

Input consists of front, left, and bottom views, which contain visible lines, hidden (dashed) lines, and noise. The first module removes dashed lines, detects visible contours, and removes unwanted edges, resulting in a clean sketch. This makes the reconstruction process more robust.

## Stage 2 — Sketch Colorization

Unlike photographs, CAD sketches only contain lines, while 3D Gaussian Splatting expects textures and colors. The authors convert the sketches into pseudo-images: the detected boundary produces a foreground mask, the interior is filled, and the result is a natural-image-like representation that can be processed by Gaussian Splatting.

## Stage 3 — Camera Pose Estimation

3DGS requires accurate camera parameters. Natural images estimate them automatically, but CAD sketches cannot. Therefore, the paper manually computes the intrinsic matrix, world-to-camera (w2c) matrix, and rotation matrix for the front, left, and bottom views. These matrices correctly position each orthographic projection in 3D space.

## Stage 4 — Gaussian Splatting Reconstruction

After preprocessing, the three processed images are fed into GaussianObject, a modified 3D Gaussian Splatting framework: three views → Gaussian initialization → optimization → 3D Gaussian model → CAD reconstruction. Unlike NeRF, 3DGS represents the object as thousands of 3D Gaussian points, each storing position, color, opacity, and covariance. These Gaussian blobs reconstruct the entire object efficiently.

# Architecture

Orthographic Drawings → Sketch Filtering → Mask Generation → Colorization → Camera Pose Calculation → 3D Gaussian Splatting → 3D CAD Model.

# Dataset

Experiments were conducted on Sub-Fusion360, derived from Fusion360. The dataset contains thousands of CAD sketches and corresponding CAD models. Only three orthographic views are used.

# Experimental Results

Compared with GaussianObject and Photo2CAD, GaussianCAD achieved better reconstruction quality, higher robustness to noisy sketches, and state-of-the-art reconstruction accuracy. The qualitative comparisons in Figure 1 show noticeably cleaner and more accurate reconstructions across different CAD objects.

# Strengths

- No ground truth needed — completely self-supervised, no 3D labels

- Uses raster sketches — no vector drawings, practical for industry

- Robust to noise — traditional methods fail, GaussianCAD still reconstructs

- Uses only three views — unlike NeRF, requires only front, left, and bottom

- Uses existing 3DGS — no need to build an entirely new reconstruction network

- High-quality visualization — 3DGS provides realistic visualization and efficient rendering of reconstructed models

# Limitations

## 1. Requires Three Orthographic Views

Cannot reconstruct from one image, a QR code, or a point cloud.

## 2. No Semantic Understanding

It reconstructs geometry only. It cannot answer a request such as "Make this chair suitable for hospital use."

## 3. No CAD Editing

Produces a 3D reconstruction, not editable CAD operations.

## 4. No Natural Language Interface

Input is a sketch only. No LLM is involved.

## 5. No Compact Representation

Entire sketches must always be transmitted. There is no QR encoding.

## 6. Manual Camera Calibration

Camera poses are manually computed, which makes large-scale deployment difficult.

# How Your Method is Better

Your Proposed Pipeline: CAD Model → 3D Slicing → Feature Extraction → QR Encoding → Natural Language Prompt → LLM → Synthetic CAD Model.

| GaussianCAD | Your MTP |
| --- | --- |
| Uses three orthographic drawings | Uses QR-encoded CAD features |
| Requires sketch preprocessing | Uses directly extracted geometric features |
| Manual camera pose estimation | No camera calibration required |
| Reconstructs geometry | Reconstructs and redesigns geometry |
| No semantic understanding | Understands prompts like "hospital chair" |
| No natural language support | Uses LLM-guided generation |
| Large raster inputs | Compact QR representation |
| Reconstruction only | Context-aware synthetic CAD generation |

# Research Gap

GaussianCAD effectively reconstructs 3D geometry from orthographic sketches without requiring vector data or ground-truth supervision. However, it remains focused on geometric reconstruction. It does not encode object semantics, support natural-language-driven modifications, or provide a lightweight mechanism for storing and sharing design information. Your MTP addresses these gaps by extracting CAD features into a QR code and combining them with LLM-based reasoning to generate application-specific synthetic CAD models while preserving the original design intent.

# Overall Summary

GaussianCAD introduces a novel self-supervised approach that adapts 3D Gaussian Splatting for CAD reconstruction from raster orthographic sketches. By reformulating CAD reconstruction as a sparse-view reconstruction problem, it removes the dependency on vector sketches and 3D ground truth while achieving strong robustness to noisy inputs. Nevertheless, it is limited to geometry reconstruction and lacks semantic reasoning, compact feature encoding, and natural-language interaction. Your proposed MTP extends this direction by introducing QR-based feature encoding and LLM-guided context-aware CAD generation, enabling not only reconstruction but also intelligent redesign of CAD models.

## Note on an Additional Paper Considered

A paper titled "Generative AI as a Tool for Transforming Digital Information into 3D Geometry" is not recommended as a primary literature review paper. Although it discusses QR codes, it focuses on 3D-printable QR code generation for additive manufacturing rather than CAD representation, reconstruction, or AI-driven CAD synthesis. It is more suitable as a supporting reference for the QR encoding aspect of your methodology than as a core paper for your literature survey.